Mechanical basin mixer

Tall single hole, single control mixer with ceramic cartridge

Mixer with swivelling, removable spout

Reference: 2871T1

Specification description

Deck-mounted tall mechanical basin mixer.

Single hole, single control with tall swivelling spout H. 180mm, L. 180mm.

Mechanical mixer supplied with 1 disposable, removable spout.

Ø 35mm ceramic cartridge with pre-set maximum temperature limiter.

Body and spout with smooth interiors (reduced bacterial development).

No risk of cross flow between the hot and cold water.

No non-return valves on the inlets.

Thermal shocks are easy to undertake without removing the control lever or shutting off the cold water supply.

Flow rate regulated at 5 lpm at 3 bar.

Hygiene control lever L. 185mm for touch-free control.

Chrome-plated brass body and base.

F3/8" PEX flexibles .

Fixing reinforced by 2 stainless steel rods.

Mechanical mixer ideal for healthcare facilities, retirement and care homes, hospitals and clinics.

Single control mixer suitable for people with reduced mobility.

Mixer with 30-year warranty.