TEMPOGENOU time flow tap

Knee-operated, Ø 15mm conical inlet

Straight for in-line inlet

Reference: 73544715

Specification description

Knee-operated time flow basin tap:

Time flow ~7 seconds.

Flow rate pre-set at 3 lpm at 3 bar, can be adjusted from 1.4 to 6 lpm.

Reinforced control lever.

Solid brass wall plate with reinforced fixing via 4 stainless steel screws.

Cold or mixed water supply.

Chrome-plated solid brass body M½".

Straight for in-line Ø 15mm conical inlet.

30-year warranty.