Mechanical basin mixer

Tall, single hole, single hole mixer with ceramic cartridge

Removable, swivelling, stainless steel spout

Reference: 2870T1

Specification description

Deck-mounted mechanical basin mixer.

Single hole, single control basin mixer with high, swivelling spout H. 180mm.

Mixer supplied with 2 removable stainless steel spouts.

Ø 35mm ceramic cartridge with pre-set maximum temperature limiter.

Body and spout with smooth interiors (reduced bacterial development).

No risk of cross flow between hot and cold water.

No non-return valves on the inlets.

Thermal shocks are easy to undertake without removing the control lever or shutting off the water supply.

Flow rate limited to 5 lpm at 3 bar.

Hygiene control lever L. 185mm requires no manual contact.

Body and base in chrome plated brass.

PEX flexibles F3/8".

Fixing reinforced by 2 stainless steel rods.

Mixer ideal for healthcare facilities, retirement and care homes, hospitals and clinics.

Mixer suitable for people with reduced mobility.

30-year warranty.