Mechanical mixer for sinks or troughs

Single control single hole tall mixer with ceramic cartridge

Mechanical mixer with swivelling spout H.165 L.140

Reference: 2564T1

Specification description

Deck-mounted mechanical basin mixer.

Single control single hole mixer with swivelling spout H. 165mm L. 140mm.

Tall mixer with BIOSAFE hygienic outlet suitable for fitting a BIOFIL point-of-use filter.

Ø 35mm classic ceramic cartridge with pre-set maximum temperature limiter.

Body and spout with smooth interiors (reduces bacterial development).

No risk of cross flow between the hot and cold water.

No non-return valves on the inlets.

Thermal shocks are easy to undertake without removing the control lever or shutting off the cold water.

Flow rate regulated at 5 lpm.

Chrome-plated body, base and spout.

Hygiene control lever L. 185mm requires no manual contact.

PEX flexibles F3/8".

Fixings reinforced by 2 stainless steel rods and back-nuts.

Mechanical mixer ideal for healthcare facilities, retirement and care homes, hospitals and clinics.

Single hole mixer suitable for people with reduced mobility.

Mixer with 30-year warranty.