TEMPOMIX shower mixer - lead-free

Single control time flow, lead-free

Recessed, stainless-steel wall plate 160x160mm

Reference: 790425LF

Specification description

Recessed time flow shower mixer:

Stainless-steel wall plate 160 x 160.

Standard recessed waterproof housing.

Recessing depth adjustable from 28 to 48 mm.

TEMPOMIX 3/4" single control shower mixer.

Ligature-resistant collar.

Temperature control and operation via the control.

Maximum temperature limiter (can be adjusted by the installer).

Time flow ~30 seconds, soft-touch operation.

Flow rate 12 lpm at 3 bar.

M1/2" connectors for Ø 15mm copper tails.

Integrated non-return valves and filters.

Chrome-plated metal control. Lead-free.

30-year warranty.

This sanitary ware model is not supplied with recessed waterproof housing, the installer must ensure the impermeability of recessed housing as well as draining any potential leaks and condensation (see installation guide).