BIOSAFE mechanical basin mixer - lead-free version

Single hole mechanical mixer with no spout H.95 L.110, lead-free

No pop-up waste

Reference: 2621LF

Specification description

Deck-mounted mechanical basin mixer, lead-free version.

Single hole mechanical mixer H.95 L.110.

No spout: hygienic flow straightener integrated into the body.

Securitouch thermal insulation prevents burns.

Ø 35mm ceramic cartridge with pre-set maximum temperature limiter.

Thermal shocks are possible without removing the control lever or shutting off the cold water.

Mixer body has smooth interior and low water volume (reduces bacterial development).

Flow rate pre-set at 4 lpm.

Solid control lever.

No pop-up waste.

F3/8" rotating PEX flexibles.

Fixing reinforced by two stainless-steel rods.

Mechanical mixer ideal for healthcare facilities, retirement and care facilities, hospitals and clinics.

Single hole mixer suitable for people with reduced mobility.

Mixer with 30-year warranty.