BIOSAFE mechanical basin mixer - lead-free version

Single hole mechanical mixer H.100 L.110 - lead-free

Solid control lever

Reference: 2921TLF

Specification description

Deck-mounted mechanical basin mixer, lead-free.

Single hole mixer with spout H.100 L.110 fitted with a BIOSAFE hygienic outlet.

Outlet suitable for fitting a BIOFIL terminal filter.

Ø 35mm ceramic cartridge with pre-set maximum temperature limiter.

Spout with internal tube with smooth interior and very low water volume.

The speed of the water inside this tube reduces biofilm development considerably.

Thermal shocks are possible without removing the control lever or shutting off the cold water supply.

Body with very low water volume (reduces bacterial development).

Flow rate pre-set at 5 lpm.

Solid control lever.

No pop-up waste.

F3/8" PEX flexibles.

Fixing reinforced by 2 stainless-steel rods.

Mechanical mixer ideal for healthcare facilities, retirement and care facilities, hospitals and clinics.

Single hole mixer suitable for people with reduced mobility.

No risk of cross flow between the hot and cold water.

No non-return valves on the inlets.

Mixer with 30-year warranty.